MODULE 2

REGULATORY &
PERMIT COMPLIANCE GUIDE

Every License, Permit, and Inspection Mapped

_______________________________________________________

ARMAGEDDON TREATS

Los Angeles County + City of Santa Monica Jurisdiction

Mobile Food Facility (MFF) -- Moderate Risk Classification

Fiscal Year 2025-2026 Fee Schedule

# 1. Regulatory Landscape Overview

Operating a mobile food facility in Santa Monica requires navigating three overlapping layers of government jurisdiction: (1) Los Angeles County Department of Public Health (LACDPH) for health permits and food safety, (2) the City of Santa Monica for business licensing and vendor permits, and (3) the State of California for tax registration and food code compliance. Each layer has its own application process, fee schedule, and renewal cycle. Missing any single permit can shut down the entire operation.

| Jurisdiction | Primary Authority | What They Control | Key Document |
| --- | --- | --- | --- |
| LA County | Dept. of Public Health (LACDPH) | Health permits, vehicle inspection, commissary verification, food safety | MFF Health Permit |
| City of Santa Monica | Finance Dept. / Farmers Market Office | Business license, vendor permit, market stall access, packaging ordinance | Business License + Vendor Permit |
| State of California | CDTFA / CRFC | Sales tax collection, Retail Food Code classification | Seller's Permit |
| Federal | SBA / IRS | Loan guarantee, tax ID, veteran certification | EIN / DD-214 |

# 2. Layer 1: Los Angeles County Public Health

## 2.1 Facility Classification

Under the California Retail Food Code (CRFC), as modified by SB 972, mobile food operations are classified based on the level of food preparation performed. Because Armageddon Treats involves active food preparation -- spinning heated sugar into cotton candy, shaving ice blocks, and dispensing flavored syrups -- the operation triggers the Moderate Risk Mobile Food Facility (MFF) classification.

| Classification | Description | Annual Permit Fee | Applies to Armageddon Treats? |
| --- | --- | --- | --- |
| CMFO, Low Risk | Prepackaged non-hazardous foods only | $126 | No |
| CMFO, Moderate Risk | Limited food prep (e.g., shaved ice from pushcart) | $299 | Possibly, if using a pushcart only |
| MFF, Low Risk | Prepackaged foods from a vehicle | $325 | No |
| MFF, Moderate Risk | Active food prep from a vehicle/trailer | $598 | YES -- primary classification |

## 2.2 Plan Check Process (Step-by-Step)

| Step | Requirement | Documents Needed | Fee |
| --- | --- | --- | --- |
| 1 | Obtain Plan Check application from LACDPH Environmental Health Division | None -- download from publichealth.lacounty.gov | None |
| 2 | Prepare detailed vehicle plans | Floor plan with dimensions, equipment layout, plumbing diagram (hot/cold water system, grey water tank), electrical diagram, ventilation plan, menu | None |
| 3 | Submit Plan Check application with plans and fee | Completed application, vehicle plans, proposed menu, equipment spec sheets with NSF/ANSI certification documentation | $347 minimum |
| 4 | LACDPH plan review (office review) | Reviewer checks plans against CRFC structural requirements | 2-6 weeks |
| 5 | Address any corrections requested by reviewer | Revised plans addressing deficiencies | May require additional fees |
| 6 | Schedule physical vehicle inspection | Fully built-out trailer, all equipment installed | Included in plan check |
| 7 | Pass vehicle inspection | Inspector verifies physical build matches approved plans | Same day |
| 8 | MFF Health Permit issued | Permit valid for one year from date of issue | $598 (annual renewal) |

## 2.3 Structural Requirements Checklist

The following specifications are non-negotiable. The trailer must meet every requirement to pass the LACDPH vehicle inspection.

Handwashing Sink  Minimum 9"W x 9"L x 5"D; warm water 100-108 deg F; soap and single-use towel dispensers mounted within arm's reach

3-Compartment Warewashing Sink  Each compartment large enough to fully submerge largest utensil; water temperature minimum 120 deg F; two integral metal drainboards

Fresh Water Tank  Adequate capacity for full day of operation (minimum 30 gallons recommended); food-grade materials; pressurized or gravity-fed delivery

Grey Water Tank  Must be at least 15% larger capacity than fresh water tank; clearly labeled; no ground discharge permitted

Mechanical Refrigeration  Minimum 12 cubic feet usable space; maintains 41 deg F or below; thermometer visible without opening

Enclosure  All food preparation areas fully enclosed with solid walls or 16-mesh screens; customer service windows maximum 216 square inches

Flooring  Smooth, nonabsorbent, easily cleanable; coved at wall junctions; no carpet

Equipment Certification  All food-contact equipment ANSI/NSF certified; all electrical appliances UL listed

Ventilation  Adequate for heat and moisture removal; hood system if cooking with grease (not required for cotton candy/shaved ice)

Lighting  Adequate illumination in all food prep and warewashing areas; shatterproof bulb covers

Waste Receptacle  Covered trash container inside unit; adequate size for full day of operation

## 2.4 Commissary Requirements

Every mobile food facility must operate from an approved commissary kitchen. This is not optional -- it is a foundational requirement of the LA County health code. The commissary is the permanent base of operations where the mobile unit returns daily.

| Commissary Function | Regulatory Requirement | Frequency |
| --- | --- | --- |
| Vehicle Storage | Overnight parking when not in operation | Daily |
| Potable Water Fill | Fill fresh water tank from approved source | Before each market day |
| Greywater Disposal | Dump into municipal sanitary sewer via approved drain | After each market day |
| Grease Disposal | Dispose in approved grease receptacle (if applicable) | As needed |
| Equipment Cleaning | Deep clean all food-contact surfaces | After each market day |
| Food Storage | Store raw ingredients in temperature-controlled environment | Daily |
| Vehicle Cleaning | Floor wash-down using industrial floor drains | Weekly minimum |

The signed "Verification of Proper Food Vehicle Storage" form from the commissary operator must be submitted to LACDPH before the health permit is issued. Health inspectors conduct random audits to verify active commissary use.

## 2.5 Route Sheet Requirement

All MFF operators must maintain a current Route Sheet that documents the vehicle's daily operating locations, including address, dates, and hours of operation. This document must be kept in the vehicle at all times and presented to health inspectors upon request. Failure to maintain an accurate Route Sheet can result in permit suspension.

# 3. Layer 2: City of Santa Monica

## 3.1 Business License and Vendor Permit

| License/Fee | Amount | Renewal | Notes |
| --- | --- | --- | --- |
| Business License Processing | $82-$100 | Annual | Base regulatory fee |
| Vendor Permit (regulatory) | $75-$80 | Annual | Includes CASp (ADA) compliance fee |
| Vehicle Vending Tax (Group IX) | $50/vehicle | Annual | Per vehicle or vending operation |
| TOTAL ANNUAL MUNICIPAL COST | $207-$230 | Annual | Must present valid LACDPH permit to obtain |

## 3.2 Disposable Food Service Ware Ordinance

Santa Monica enforces one of the most comprehensive packaging ordinances in the country. Non-compliance results in fines and potential market expulsion.

| Material | Status in Santa Monica | Armageddon Treats Compliance |
| --- | --- | --- |
| Expanded Polystyrene (Styrofoam) | BANNED -- all forms | Do not purchase; zero tolerance |
| Clear Plastic #6 | BANNED | Do not purchase |
| Conventional Plastic Cups | NOT COMPLIANT | Replace with PLA bioplastic or sugar cane pulp |
| PLA (Polylactic Acid) Bioplastic | COMPLIANT -- marine-degradable | Primary serving vessel (flower cups) |
| Sugar Cane Pulp / Bagasse | COMPLIANT -- compostable | Alternative for bowls and containers |
| Paper (uncoated, recyclable) | COMPLIANT | For cotton candy cones and wrapping |
| Wooden/Bamboo Utensils | COMPLIANT | For spoons (shaved ice) |

## 3.3 Farmers Market Vendor Rules

Attendance  Maximum 3 excused absences per calendar year. Beyond 3, vendor pays average historical fees regardless of reason for absence.

Setup/Breakdown  All vendors must be fully set up by market opening time and may not begin breakdown until official closing time.

Stall Fees  Prepared food vendors pay 12-13% of gross daily market sales (variable fee model).

Animals  No live animals permitted within 20 feet of any food storage or preparation area.

Insurance  Current Certificate of Insurance (CGL) must name City of Santa Monica as additional insured.

Signage  All vendors must display business name, pricing, and applicable health permits visibly.

Waste  Vendors are responsible for removing all waste generated during market operations.

# 4. Layer 3: State of California

## 4.1 Sales Tax Collection

California requires all food vendors to collect and remit sales tax on prepared food sales. In Santa Monica, the combined state + local sales tax rate is approximately 10.25%. As a vendor with annual sales under $100,000, you will file quarterly returns with the CDTFA. Sales tax collected is NOT your revenue -- it is held in trust for the state and must be remitted on schedule.

## 4.2 California Food Handler Card

Anyone handling food in California must possess a valid California Food Handler Card. This requires completing an approved food safety course and passing an exam. Cost: $10-$15 online. Valid for 3 years. The owner and any future employees must each hold their own card before handling food.

# 5. Master Permit and License Tracker

Use this table to track the status of every required permit and license. Keep a physical copy in the trailer at all times.

| Permit/License | Issuing Authority | Cost | Renewal | Status |
| --- | --- | --- | --- | --- |
| EIN | IRS | Free | Permanent |  |
| DBA Registration | LA County Clerk | $26 + publication | 5 years |  |
| Seller's Permit | CA CDTFA | Free | Permanent |  |
| Food Handler Card | Approved provider | $10-$15 | 3 years |  |
| MFF Health Permit | LACDPH | $598/year | Annual |  |
| Plan Check Fee | LACDPH | $347+ | One-time |  |
| Commissary Agreement | Commissary operator | $300-$500/mo | Per contract |  |
| SM Business License | City of Santa Monica | $82-$100/year | Annual |  |
| SM Vendor Permit | City of Santa Monica | $75-$80/year | Annual |  |
| Vehicle Vending Tax | City of Santa Monica | $50/year | Annual |  |
| CGL Insurance | Insurance broker | $120/mo | Annual policy |  |
| Commercial Auto Insurance | Insurance broker | $156-$260/mo | Annual policy |  |
| FM RFP Approval | SM Farmers Market Office | N/A (variable stall fees) | Annual/per RFP cycle |  |

ANNUAL COMPLIANCE CALENDAR January: Renew SM Business License and Vendor Permit. File prior-year taxes. March: Farmers Market RFP window (if seeking new/additional markets). June/July: LACDPH Health Permit renewal (date depends on original issue). Quarterly: File CDTFA sales tax returns. Monthly: Pay commissary rent, insurance premiums, SBA loan payment. Daily: Maintain Route Sheet in vehicle. Keep all permits physically in trailer.